# DOUBLE JACKETED GASKET

### Metal-Jacketed Sealing Solution for High-Temperature and Narrow Flange Applications

## TECHNICAL DATASHEET

### Product Description

INMARCO Double Jacketed Gaskets are semi-metallic sealing solutions consisting of a soft filler material completely enclosed within a metallic outer jacket. This construction combines the compressibility and recovery characteristics of non-metallic fillers with the strength, blowout resistance, and durability of metallic materials.

The metal jacket protects the filler from process media while providing excellent mechanical strength and corrosion resistance. These gaskets are particularly suitable for narrow flange widths, rough or uneven flange faces, and applications involving high temperatures, thermal cycling, and aggressive process conditions.

Double Jacketed Gaskets are widely used in heat exchangers, boilers, pressure vessels, pumps, valves, steam systems, and process equipment where reliable sealing performance is essential.

## Key Features

● Semi-Metallic Construction
Combines a metallic outer jacket with a resilient soft filler to provide excellent sealing performance.

● Excellent Blowout Resistance
Metal jacket provides structural integrity and resistance against media penetration and gasket failure.

● Superior Conformability
Soft filler materials conform to flange irregularities, scratches, and surface imperfections.

● Wide Material Availability
Available in various jacket and filler combinations to suit different operating conditions.

● Suitable for Narrow Flanges
Ideal for applications where flange sealing width is limited.

● Custom Manufacturing Options
Available in round, oval, oblong, ribbed, partitioned, and special engineered configurations.

## Applications

### Typical Equipment

• Heat exchangers
• Pressure vessels
• Boilers
• Autoclaves
• Pumps
• Valve bonnets
• Blast furnaces
• Steam lines
• Gas mains
• Process piping systems

### Industry Applications

• Petrochemical industry
• Oil & gas industry
• Chemical processing
• Power generation
• Food processing
• Pharmaceutical manufacturing
• General industrial services

## Technical Specifications

| Property | Value |
| --- | --- |
| Operating Temperature | -200°C to +550°C* |
| Pressure Range | Vacuum to 150 bar* |
| Construction | Double Jacketed |
| Gasket Type | Semi-Metallic |
| Flange Surface Compatibility | Rough, uneven, and narrow flange faces |
| Shape Options | Round, Oval, Elliptical, Oblong, Segmental |
| Special Designs | Ribbed, Partitioned, Corrugated Double Jacketed, Welded Types |

* Depending on jacket and filler material selection.

## Material Options

### Outer Jacket Materials

Material

Stainless Steel 304

Stainless Steel 316

Monel®

Inconel®

Copper

Nickel

### Filler Materials

Material

Flexible Graphite

Ceramic Fiber

PTFE

Non-Asbestos Fiber

## Applicable Standards

Standard

ASME B16.20

DIN EN 1514-6

API 601

## Performance Benefits

• Excellent resistance to blowout and process media penetration.

• Metallic jacket provides outstanding mechanical strength and corrosion resistance.

• Soft filler ensures excellent recovery and adaptability to flange surface irregularities.

• Suitable for applications involving thermal cycling and fluctuating operating conditions.

• Handles moderate to high operating pressures with reliable sealing performance.

• Ideal for narrow flange widths where conventional gaskets may not perform effectively.

• Economical solution for demanding sealing applications requiring durability and long service life.

• Available in multiple material combinations to suit aggressive chemicals, steam, hydrocarbons, and high-temperature media.

• Performs reliably where conventional soft gaskets may fail due to mechanical abuse, temperature exposure, or pressure fluctuations.

## Additional Construction Options

• Single Jacketed Gaskets

• Double Jacketed Gaskets

• Corrugated Double Jacketed Gaskets

• Welded Ribbed Gaskets

• Partitioned Heat Exchanger Gaskets

• Special Custom-Manufactured Designs