# GM 300Z

### High-Purity Graphite Molded Rings with Exceptional Oxidation Resistance

### Product Description

INMARCO GM 300Z is a high-efficiency, ultra-low oxidation graphite molded ring designed for valve stem and seat sealing applications. Each ring is manufactured from flexible graphite foil with ultra-high purity (≥99.85%) and extremely low detrimental material content.

The rings incorporate advanced oxidation inhibitors and corrosion-resistant properties, meeting stringent requirements for nuclear and high-temperature service applications. They are precisely sized, density-controlled, and manufactured without binders, elastomers, or fillers, ensuring superior sealing reliability and long service life.

GM 300Z is specifically developed for critical valve applications handling high-temperature steam, hydrocarbons, heat transfer fluids, and other demanding process media where graphite purity and oxidation resistance are essential.

## Key Features

● Ultra-High Purity Graphite
Manufactured from flexible graphite foil with purity levels of ≥99.85% and extremely low sulfur content.

● Advanced Oxidation Resistance
Built-in oxidation inhibitors provide exceptional resistance to oxidation at elevated temperatures.

● Nuclear Industry Compliance
Designed to meet stringent nuclear industry requirements for valve stem and seat sealing applications.

● Binder-Free Construction
Free from elastomers, fillers, and binders for maximum thermal stability and chemical compatibility.

● Precision Manufactured
Density-controlled and accurately machined for consistent sealing performance.

● Low Detrimental Material Content
Minimizes decomposition and degradation during prolonged high-temperature operation.

## Applications

### Suitable For

• Valve stem sealing systems
• Valve seat sealing systems
• Rising stem valves
• Rotary valves
• Class 150–1500 valves
• Higher pressure classes upon technical review

### Service Media

• Superheated steam
• Saturated steam
• Hydrocarbons
• Heat transfer fluids
• Fuel oil and lubricating oil
• Flue gases
• Hot dry ash
• Emissive fluids
• Chemicals and dyes
• Non-oxidizing liquids and gases
• Nuclear power generation process fluids

## Technical Specifications

| Property | Value |
| --- | --- |
| pH Range | 0 – 14 |
| Maximum Temperature | 650°C |
| Maximum Pressure | 250 bar |
| Size Availability | According to chamber size |
| Graphite Purity | ≥99.85% |
| Sulfur Content | < 300 ppm |
| Chloride Content | ≤ 10 ppm |
| Fluoride Content | ≤ 10 ppm |
| Halogen Content | ≤ 40 ppm |
| Density | 1.4 to 1.8 gm/cm³ |
| Oxidation Inhibitor | Built-in |
| Construction | Flexible Graphite Molded Ring |

## Thermal Properties

| Property | Value |
| --- | --- |
| Thermal Conductivity (Axial) | 400 W/mK |
| Thermal Conductivity (Radial) | 6 W/mK |
| Coefficient of Friction (Steel) | 0.05 |

## Performance Benefits

• Ultra-high purity graphite construction ensures reliable sealing performance in critical applications.

• Exceptional oxidation resistance extends service life in high-temperature environments.

• Low sulfur and halogen content make it suitable for nuclear and sensitive process applications.

• Excellent thermal conductivity promotes efficient heat dissipation.

• Low friction coefficient minimizes stem wear and operating torque.

• Suitable for both rotary and rising-stem valve applications.

• Compatible with high-pressure steam, hydrocarbons, and a wide range of industrial process media.

• Fire-safe design suitable for safety-critical installations.

• Available for valve pressure classes from Class 150 to Class 1500; consult INMARCO for Class 2500 applications.

## Certifications & Compliance

• Graphite foil tested and certified as per Shell MESC SPE 85/203

• Oxidation performance verified using gravimetric analysis

• Typical oxidation rate: 2.6% weight loss per hour at 670°C for 4 hours

• EN 10204 Type 2.2 Material Certificate available upon request