# GM 310C

### High Extrusion Resistance Pressure Seal Ring

### GM 310C

INMARCO Style GM 310C Graphite Pressure Seal Rings are manufactured from ultra-high-purity graphite ribbon without additives or binders. Designed specifically for pressure seal valves, these rings provide outstanding sealing performance under severe temperature and pressure conditions.

The sealing rings are fitted with SS or Inconel end caps on both the top and bottom surfaces to provide exceptional extrusion resistance. They are available in a variety of configurations, including rectangular, U-shape, and I-shape constructions, allowing customization to suit different valve designs and service conditions.

The combination of high-purity graphite and robust metallic reinforcement ensures excellent thermal stability, low oxidation rates, and long service life in demanding industrial applications.

## Key Features

● Manufactured from ultra-high-purity graphite ribbon with a minimum purity of 99.85%.

● Stainless Steel or Inconel end caps provide exceptional resistance to extrusion under high-pressure conditions.

● Low detrimental material content minimizes decomposition at elevated temperatures.

● Exhibits extremely low oxidation rates, ensuring long-term performance in high-temperature applications.

● Available with optional Inconel or SS wire mesh reinforcement for severe service conditions and very high-pressure applications.

● Offered in multiple configurations, including rectangular, U-shape, and I-shape constructions to suit various valve designs.

● Suitable for critical pressure seal valve applications requiring reliable sealing performance and extended service life.

## Service Media

● Superheated Steam

● Saturated Steam

● Hot Dry Ash

● Flue Gases

● Hydrocarbons

● Fuel Oils

● Lubricating Oils

● Thermal Fluids

● Dyes and Chemicals

● Emissive Fluids

● All Non-Oxidizing Liquids and Gases

## Typical Applications

● Pressure Seal Valves

● High-Pressure Steam Systems

● Power Generation Facilities

● Petrochemical Plants

● Refining Operations

● High-Temperature Process Equipment

## Technical Specifications

### Technical Properties

| Property | Value |
| --- | --- |
| pH Range | 0 – 14 |
| Maximum Pressure | 425 Bar |
| Temperature Range | As Per Graphite Grade (Contact INMARCO for Details) |
| Size | Based on Valve Design |
| Compatibility | As Per Graphite Grade |
| Graphite Purity | ≥ 99.85% |
| Construction Options | Rectangular, U-Shape, I-Shape |
| End Cap Materials | SS / Inconel |
| Optional Reinforcement | SS / Inconel Wire Mesh |

## Certification & Testing

● Graphite foil tested and certified in accordance with Shell MESC SPE 85/203.

● Oxidation performance verified using Thermogravimetric Analysis (TGA).

● Recorded weight loss of only 2.6% after exposure to 670°C for 4 hours.

## Performance Benefits

● Exceptional resistance to extrusion in high-pressure valve applications.

● Ultra-high-purity graphite ensures superior thermal stability and sealing performance.

● Low oxidation rate contributes to extended service life at elevated temperatures.

● Suitable for severe operating conditions involving high pressure and temperature.

● Optional wire mesh reinforcement provides enhanced mechanical strength.

● Available in specialized graphite grades for extreme temperature applications.

● Reliable sealing performance reduces maintenance requirements and improves operational safety.