# HY 105

### Engineered for Mechanical Stability in High-Load Equipment

## Style HY 105

HY 105 is a high-performance gland packing manufactured from a dense braid of Aramid Yarn corners and PTFE Yarn faces, reinforced with a resilient Aramid fibre core. The materials are treated with advanced lubricants to reduce friction and enhance thermal resistance, ensuring smooth operation and extended service life.

This packing is designed for demanding industrial environments, offering excellent chemical resistance, high-pressure tolerance, and dimensional stability. It is ideal for use in pumps, valves, mixers, and reactors across chemical, fertilizer, and process industries.

### Also Available in Economical Version

This product is also available in an economical version as HY 105E, offering cost-effective sealing solutions while maintaining reliable performance standards.

## Key Features

● Unique combination of Aramid Yarn corners and PTFE Yarn faces designed for enhanced sealing performance.

● Reinforced with a high-tensile Aramid fiber core, providing exceptional mechanical strength and dimensional stability.

● PTFE faces are treated with inert, high-temperature lubricants to ensure smooth, low-friction operation.

● Aramid corners are lubricated with antifrictional fluoropolymer dispersion, helping prevent corner failures in reciprocating equipment.

● Delivers reliable sealing performance in chemically aggressive and mechanically demanding environments.

● Excellent resistance to wear, extrusion, and mechanical stress.

## Applications

● Centrifugal pumps

● Reciprocating pumps

● Plunger pumps

● Valves

● Mixers

● Agitators

● Reactors

● Chemical processing equipment

● Fertilizer production facilities

● Process industry applications

● Applications involving slurries, pigments, and synthetic compounds

## Technical Specifications

### Technical Properties

| Property | Rotating | Reciprocating |
| --- | --- | --- |
| pH Range | 0 – 14 | 0 – 14 |
| Temperature Range | -200°C to +300°C | -200°C to +300°C |
| Pressure (Bar) | 100 | 200 |
| Velocity (m/s) | 25 | 10 |
| Available Sizes | 3 sq mm to 35 sq mm | 3 sq mm to 35 sq mm |

## Performance Benefits

● Engineered for long-lasting sealing performance in dynamic operating environments.

● Combines the strength of Aramid fiber with the low-friction characteristics of PTFE.

● Delivers smooth operation while protecting shafts, sleeves, and equipment surfaces from wear.

● Provides excellent dimensional stability under varying pressure and temperature conditions.

● Designed to perform reliably across a broad range of industrial applications.

● Superior chemical resistance against a wide range of process media.

● Maintains sealing integrity under demanding mechanical loads.

● Reduces maintenance frequency and extends equipment service life.

● Excellent resistance to extrusion and packing deformation.

● Supports reliable operation in critical chemical and fertilizer industry applications.

## Service Media

● Slurries

● Pigments

● Synthetic compounds

● Industrial process fluids

● Chemical media

● Fertilizer process chemicals

● Water-based solutions

● General industrial media

### 7. COMPLIANCE

- FDA Certified