# TECHNICAL DATASHEET

# KAMMPROFILE GASKET

### High-Performance Serrated Metal Gasket for Demanding Industrial Applications

## 1. PRODUCT OVERVIEW

INMARCO Kammprofile Gaskets are high-performance metallic gaskets designed with a precision-machined serrated metal core and soft sealing layers bonded to both sealing surfaces. This unique construction combines the mechanical strength of metal with the superior sealing characteristics of soft facing materials, delivering exceptional sealing reliability across a wide range of pressure and temperature conditions.

The concentric serrations concentrate gasket loading and create highly effective sealing barriers while minimizing flange stress and bolt load requirements. Kammprofile gaskets offer excellent resistance to blowout, thermal cycling, pressure fluctuations, and radial shear forces, making them ideal for critical industrial applications.

The reusable metal core allows refurbishment by replacing only the soft sealing layers, significantly reducing maintenance costs and extending service life.

## 2. KEY FEATURES

- Precision-machined serrated metal core construction

- Soft sealing layers bonded to both sealing surfaces

- Excellent sealing performance under low seating loads

- High resistance to blowout and pressure fluctuations

- Outstanding performance under thermal cycling conditions

- Reusable metal core reduces lifecycle costs

- Excellent resistance to radial shear forces

- Suitable for standard and custom flange configurations

- Available with graphite, PTFE, mica, and specialty facing materials

- Compatible with a wide range of metallic core materials

## 3. PERFORMANCE ADVANTAGES

- Provides exceptional sealing reliability across varying pressure and temperature conditions

- Requires lower bolt loads compared to many traditional metallic gaskets

- Reduces flange distortion and minimizes equipment stress

- Maintains sealing integrity during thermal expansion and contraction cycles

- Offers excellent recovery characteristics under fluctuating operating conditions

- Minimizes leakage risks in critical process services

- Enables refurbishment through replacement of facing materials while retaining the metal core

- Reduces maintenance frequency and overall operating costs

- Suitable for severe service environments requiring leak-tight performance

- Increasingly preferred over spiral wound and metal-jacketed gasket designs in critical applications

## 4. TECHNICAL PROPERTIES

| Property | Specification |
| --- | --- |
| Construction | Serrated Metal Core with Soft Facing Layers |
| Pressure Rating | Up to 250 bar (application dependent) |
| Temperature Range | Cryogenic to +1000°C (depending on facing material) |
| Facing Materials | Graphite, PTFE, Mica, Specialty Fillers |
| Core Materials | SS304, SS316, SS321, SS347, Inconel, Monel, Titanium, Copper and Other Alloys |
| Flange Standards | ASME B16.5, ASME B16.47, EN, DIN, Custom |
| Reusability | Metal Core Reusable After Refacing |
| Blowout Resistance | Excellent |
| Thermal Cycling Resistance | Excellent |
| Radial Shear Resistance | Excellent |

## 5. TEMPERATURE & PRESSURE LIMITS

| Parameter | Typical Range |
| --- | --- |
| Operating Pressure | Up to 250 bar |
| Operating Temperature | Cryogenic to +1000°C |
| Seating Load Requirement | Low |
| Thermal Cycling Capability | Excellent |

Actual operating limits depend on selected core and facing materials.

## 6. MATERIAL OPTIONS

### Core Materials

- Stainless Steel 304

- Stainless Steel 316

- Stainless Steel 321

- Stainless Steel 347

- Inconel

- Monel

- Titanium

- Copper

- Other Special Alloys

### Facing Materials

- Flexible Graphite

- PTFE

- Mica

- Specialty Soft Sealing Materials

## 7. STANDARDS & DIMENSIONS

| Description | Availability |
| --- | --- |
| ASME B16.5 Flanges | Available |
| ASME B16.47 Flanges | Available |
| DIN Flanges | Available |
| EN Flanges | Available |
| Heat Exchanger Gaskets | Available |
| Custom Profiles | Available Upon Request |

## 8. TYPICAL APPLICATIONS

- Heat Exchangers

- Pressure Vessels

- Pumps

- Valves

- Reactors

- Petrochemical Processing Equipment

- Refinery Applications

- Power Generation Facilities

- Nuclear Power Plants

- Marine Applications

- Chemical Processing Equipment

- High-Pressure Piping Systems

- Custom Process Equipment

## 9. SERVICE MEDIA

Suitable for:

- Steam

- Hydrocarbons

- Oils

- Process Chemicals

- Solvents

- Acids

- Alkalis

- Gases

- Thermal Fluids

- General Industrial Process Media

Material selection should be based on process media compatibility and operating conditions.

## 10. PERFORMANCE BENEFITS

- Exceptional sealing performance under high pressure and temperature conditions

- Lower seating stress requirements compared to many metallic gasket designs

- Excellent resistance to thermal cycling and pressure fluctuations

- Superior blowout resistance for enhanced operational safety

- High resistance to radial shear and mechanical loading

- Reusable metal core significantly reduces maintenance costs

- Wide range of metallic core and facing material combinations

- Suitable for both standard flanges and custom equipment

- Reliable sealing performance in critical industrial applications

- Long service life with reduced plant downtime

## DISCLAIMER

All technical information, recommendations, and performance data provided in this technical datasheet are based on current knowledge, laboratory testing, and industrial experience. Actual product performance may vary depending on operating conditions, equipment design, installation practices, process media, pressure, temperature, and application environment. Final product selection and suitability should be determined by the user based on the specific service conditions and application requirements.