# OR 151

## Duplex Braided Vegetable Fibre Packing with Proprietary PTFE Dispersion

INMARCO Style OR 151 is a soft vegetable fibre (flax) yarn duplex braided packing impregnated with a special proprietary PTFE-based dispersion incorporating performance-enhancing additives. During manufacturing, each yarn is individually impregnated with the dispersion before braiding, followed by a final surface treatment with PTFE dispersion and special additives to maximize sealing efficiency and service life.

Manufactured from vegetable fibre naturally compatible with water-based applications, OR 151 provides excellent resistance to deterioration in wet environments. The PTFE-based treatment imparts outstanding anti-friction properties, minimizing sleeve and shaft wear while ensuring reliable sealing performance in pumps and marine equipment.

## Key Features

● Duplex braided from high-quality flax (vegetable) fibre for excellent flexibility and sealing performance.

● Each yarn is individually impregnated with proprietary PTFE-based dispersion before braiding.

● Final surface treatment with PTFE dispersion and special additives enhances lubrication and sealing efficiency.

● Highly compatible with water-based applications due to the natural properties of vegetable fibre.

● Excellent anti-friction characteristics prevent scoring and wear on shafts and sleeves.

● Good resistance to deterioration in wet and submerged operating conditions.

● Reliable sealing performance with low maintenance requirements.

● Easy installation and adjustment.

## Applications

● Pumps

● Water wheel glands

● Stern glands

● Rudder posts

● Marine equipment

● Water handling systems

● Utility service equipment

## Service Media

● River water

● Sea water

● Sewage water

● Surface water

● Industrial water systems

● General water-based media

## Technical Specifications

| Properties | Rotating |
| --- | --- |
| pH Range | 4 – 10 |
| Temperature (°C) | Up to 150 |
| Pressure (Bar) | 80 |
| Velocity (m/s) | 5 |
| Size | 3 sq mm to 35 sq mm |

## Performance Benefits

● Excellent compatibility with water and aqueous media.

● Superior anti-friction properties due to proprietary PTFE dispersion.

● Prevents sleeve and shaft scoring, extending equipment life.

● Good resistance to wear and deterioration in wet environments.

● Enhanced sealing performance through dual-stage PTFE impregnation.

● Reliable operation in marine and water-service applications.

● Easy installation and maintenance.

● Economical and dependable solution for general water-handling equipment.

● Long service life with consistent sealing performance

Note: OR 151 is particularly suitable for water-handling, marine, and utility applications where low friction, shaft protection, and reliable sealing performance are critical.