# Style PE 102

Precision Engineered for Chemical Resistance and Thermal Stability

### Style PE 102

Style PE 102 is a premium inter-braided packing manufactured from expanded PTFE and graphite fibre. This advanced molecular blend combines the exceptional chemical resistance of PTFE with the thermal conductivity and lubricating properties of graphite, delivering superior sealing performance compared to conventional PTFE-graphite packings.

The packing utilizes high-performance K-fibre technology from WL Gore Inc., with graphite molecularly bonded during the polymerization process. This unique construction prevents graphite migration and disintegration, maintains continuous lubrication, and ensures long-term structural integrity. PE 102 is particularly suitable for pumps operating without cooling water, providing reliable sealing, reduced friction, and enhanced equipment protection.

## Key Features

● Manufactured from an advanced inter-braided blend of expanded PTFE and graphite fibre.

● Graphite is molecularly bonded during polymerization, preventing separation and ensuring long-term performance.

● Exceptional chemical resistance against acids, alkalis, hydrocarbons, solvents, and aggressive process media.

● High thermal conductivity promotes efficient heat dissipation and cooler equipment operation.

● Low coefficient of friction minimizes shaft and sleeve wear.

● Low thermal expansion maintains sealing integrity under fluctuating temperatures.

● Continuous graphite lubrication prevents hardening and maintains packing flexibility over time.

● Anti-static characteristics help prevent static charge build-up in sensitive process environments.

● Flexible construction allows easy cutting, installation, and maintenance.

● Ideal for pumps operating without external cooling water systems.

## Applications

● Centrifugal and rotary pumps.

● Control valves and process valves.

● Reactors and agitators.

● Chemical processing equipment.

● Petrochemical and refinery services.

● Power generation facilities.

● Water treatment and industrial process systems.

● Applications involving acids, alkalis, hydrocarbons, solvents, and aggressive chemicals.

● Dynamic and static sealing applications requiring low friction and long service life.

## Technical Specifications

### Technical Properties

| Property | Value |
| --- | --- |
| pH Range | 0 – 14 |
| Temperature (°C) | -240 to +300 |
| Pressure (Bar) | 300 |
| Size | 3 sq mm to 35 sq mm |

## Performance Benefits

● Exceptional resistance to a wide range of chemicals including acids, alkalis, hydrocarbons, and solvents.

● High thermal conductivity ensures cooler operation and reduced friction in dynamic equipment.

● Superior sealing performance compared to conventional PTFE-graphite packings.

● Low thermal expansion maintains sealing effectiveness during temperature fluctuations.

● Anti-static properties improve safety in chemical and hydrocarbon processing environments.

● Molecularly bonded graphite provides continuous lubrication and prevents hardening over time.

● Reduces shaft and sleeve wear, extending equipment service life.

● Suitable for pumps operating without cooling water.

● Easy installation and maintenance reduce downtime and labor costs.

● Excellent long-term dimensional stability and sealing reliability.

## Certifications & Compliance

● Manufactured using high-performance K-Fibre Technology from WL Gore Inc.

● Suitable for use across a broad range of industrial chemical processing applications.

## Availability

Size Range: 3 sq mm to 35 sq mm