# 1. PRODUCT OVERVIEW

INMARCO Style PE 505 is a high-performance duplex braided compression packing manufactured from a specially engineered combination of Expanded PTFE, Graphite, and MoS₂ bonded at the molecular level. The advanced material structure provides excellent chemical resistance, superior heat dissipation, low friction characteristics, and high dimensional stability under dynamic operating conditions.

Designed for demanding industrial sealing applications, Style PE 505 delivers reliable sealing performance with reduced shaft wear, low leakage rates, and extended service life. Its anti-frictional composition minimizes equipment wear while supporting smooth operation in high-speed rotating and reciprocating equipment.

The dense and resilient braided construction ensures consistent sealing integrity with minimal gland adjustment requirements, making it suitable for pumps, valves, agitators, mixers, reactors, and other critical industrial equipment operating under severe service conditions.

Style PE 505 is compatible with a broad range of industrial media excluding strong oxidizers, molten alkali metals, and fluorine compounds.

# 2. KEY FEATURES

- Manufactured from Expanded PTFE, Graphite, and MoS₂ bonded at the molecular level

- Excellent chemical resistance across a wide range of industrial media

- Superior heat dissipation improves operational stability and packing life

- Low friction coefficient minimizes shaft and sleeve wear

- Dense braided construction ensures dimensional stability under dynamic conditions

- Requires minimal gland loading for effective sealing performance

- High resilience and recovery characteristics under fluctuating operating conditions

- Suitable for high-speed rotating and reciprocating equipment

- Low leakage performance with reduced maintenance requirements

- Tested and qualified as per EN16752 tightness class T1

# 3. PERFORMANCE ADVANTAGES

- Protects shafts and sleeves from wear, scoring, and erosion during continuous operation

- Maintains reliable sealing performance under high-speed dynamic service conditions

- Excellent thermal conductivity reduces heat build-up within stuffing box systems

- Reduces equipment downtime and maintenance frequency

- Provides smooth operational performance with low friction resistance

- Maintains dimensional stability under pressure and thermal cycling conditions

- Excellent adaptability to shaft movement and operational vibration

- Suitable for chemically aggressive process media applications

- Supports longer packing life and improved equipment reliability

- Enhances operational efficiency in rotating equipment systems

# 4. TECHNICAL PROPERTIES

| Property | Specification |
| --- | --- |
| Construction | Duplex braided packing |
| Material Composition | Expanded PTFE + Graphite + MoS₂ |
| pH Range | 0 – 14 |
| Temperature Range | -200°C to +300°C |
| Pressure – Rotating | Up to 100 bar |
| Pressure – Reciprocating | Up to 200 bar |
| Pressure – Static | Up to 300 bar |
| Shaft Speed – Rotating | Up to 25 m/s |
| Shaft Speed – Reciprocating | Up to 10 m/s |
| Size Range | 3 mm² to 35 mm² |
| Chemical Compatibility | Suitable for most industrial chemicals except strong oxidizers, molten alkali metals, and fluorine compounds |
| Standards Compliance | Tested as per EN16752 – Tightness Class T1 |

# 5. APPLICATIONS

- Centrifugal and rotary pumps

- Valves and control valves

- Agitators and mixers

- Reactors and process vessels

- High-speed rotating equipment

- Chemical processing industries

- Petrochemical and refinery applications

- Power generation systems

- Water and wastewater treatment systems

- General industrial sealing applications requiring low-friction performance

# 6. PERFORMANCE BENEFITS

- Long operational life under demanding industrial service conditions

- Excellent resistance to wear, friction, and thermal degradation

- Reduced shaft damage and lower equipment maintenance costs

- Consistent sealing performance with minimal leakage

- Reliable operation under dynamic and fluctuating conditions

- Suitable for continuous operation in critical process industries

- Improved equipment efficiency due to reduced friction losses

- Broad media compatibility for versatile industrial applications

# 7. TESTING & COMPLIANCE

- Tested and qualified as per EN16752

- Tightness Class T1 certified performance

- Verified for dynamic sealing applications under industrial operating conditions

# 10. DISCLAIMER

The technical information provided in this datasheet serves as a general engineering guideline. Final product selection should be based on actual operating conditions, equipment design, media compatibility, pressure, temperature, and shaft speed requirements.