# PE 508

### Versatile Packing for Dynamic and Static Applications

### PE 508

INMARCO Style PE 508 is a high-performance diagonally braided packing manufactured from pure PTFE filament yarn and impregnated with a proprietary break-in lubricant. This combination provides smooth start-up characteristics, enhanced pressure resistance, and reliable long-term sealing performance.

The packing is chemically inert, non-toxic, and dimensionally stable, ensuring safe operation and consistent sealing efficiency across a wide range of industrial applications. Its low-friction design minimizes equipment wear, reduces maintenance requirements, and delivers dependable performance in both dynamic and static sealing environments.

## Key Features

● Manufactured from high-quality pure PTFE filament yarn for exceptional chemical resistance.

● Diagonal braided construction enhances structural integrity and ensures uniform sealing pressure.

● Proprietary break-in lubricant reduces start-up friction and minimizes wear during initial operation.

● Suitable for a wide temperature range and aggressive chemical environments.

● Withstands pressures up to 350 bar for demanding industrial sealing applications.

● Dimensionally stable construction minimizes leakage and maintains sealing integrity over extended service periods.

● Non-toxic and chemically inert, making it suitable for handling sensitive and corrosive media.

## Typical Applications

● Pumps

● Agitators

● Mixers

● Reactors

● Static and Dynamic Sealing Equipment

● Chemical Processing Industries

● Water Treatment Systems

## Technical Specifications

### Technical Properties

| Property | Value |
| --- | --- |
| pH Range | 0 – 14 |
| Temperature Range | -240°C to +280°C |
| Maximum Pressure | 350 Bar |
| Available Sizes | 3 sq mm to 35 sq mm |
| Construction | Diagonally Braided Pure PTFE Filament Yarn |
| Lubrication | Proprietary Break-In Lubricant |

## Performance Benefits

● Exceptional resistance to high pressure and mechanical stress.

● Dimensionally stable construction ensures consistent sealing performance and extended service life.

● Integrated break-in lubricant enables smooth start-up and reduces equipment wear.

● Suitable for both dynamic and static sealing applications.

● Chemically inert and non-toxic composition ensures safe and clean operation.

● Low friction characteristics help protect shafts, sleeves, and equipment surfaces.

● Reduces maintenance requirements and minimizes operational downtime.

● Delivers reliable sealing performance in demanding industrial environments.